REPORT OF SURVEY

FL PANHANDLE

BAY, CALHOUN, FRANKLIN, GULF, WALTON, AND WASHINGTON COUNTIES, FLORIDA

# INTRODUCTION

Earthdata International was contracted to provide mapping services for the NW Greenway located in Bay, Calhoun, Franklin, Gulf, Walton and Washington Counties, Florida. Digital aerial imagery (Leica ADS-40) observations were made for the project. EarthData subcontracted the ground survey tasks to Terrasurv. Terrasurv utilized the Global Positioning System (GPS) to establish the control network.

# CONTROL

The control for this project consisted of several stations of the National Spatial Reference System (NSRS).

The following list summarizes the NSRS control used for this project:

Station Name		PID		H Order	V Order	Comments

APALPORT AZ MK		AS0895	   1		   2		HORIZ/BM

FLGPS 13 AZ MK		AS0900	   1		   2		HORIZ/BM

PARISH RESET		BE3768	   B		   2		HARN/BM

X 88				BE0864	   1		   2		HORIZ/BM

PANAMA CORS ARP		DE6014	   A		   -		CORS

The horizontal datum was the North American Datum of 1983, 1999 adjustment (NAD 1983 1999). The vertical datum was the North American Vertical Datum of 1988 (NAVD 1988).

# NEW STATIONS

The photo control stations were established post flight; therefore all new stations are photo-identifiable points. No permanent stations were set. There were a total of 25 new photo identities established. In addition, there were 3 temporary base points and the airborne GPS (ABGPS) base station used by the flight crew. The following is a listing of the stations surveyed for this project:

| Station Name | GPSID | USGS Quad | Description |
| --- | --- | --- | --- |
| APALPORT AZ MK | AS0895 | WEST PASS | H/Vcontrol |
| BASE | 05013AA | REDBAY | Temporary GPS base station |
| BASE | 05013AB | YOUNGSTOWN | Temporary GPS base station |
| BASE | 05013AC | DEAD LAKE | Temporary GPS base station |
| ECC STA | 05013W | JACKSON RIVER | ECC STA |
| EXTRA | 05013O | TENMILE SWAMP | ID=north tip of grass island around pole |
| FLGPS 13 AZ MK | AS0900 | FORBES ISLAND | H/V control |
| HV-01 | 05013A | DE FUNIAK SPRINGS EAST | ID=SW corner of asphalt drive at north edge of CR 280A |
| HV-02 | 05013B | PORTLAND | ID=SE corner of darker asphalt patch at south edge of Woodlawn Road |
| HV-03 | 05013C | BUNKER | ID=NW corner of dar asphalt at north edge of CR 3280 |
| HV-04 | 05013D | ROCK HILL | ID=north tip of grass at intersection of sand roads |
| HV-05 | 05013E | MILLERS FERRY | ID=Center of west end of stop bar for Icetea Road at north side of CR 264 |
| HV-06 | 05013F | VERNON | ID=NE corner of wooden dock |
| HV-07 | 05013G | RED HEAD | ID=center (top) of east edge of westerly drainage structure under field road on south side of SR 20 |
| HV-08 | 05013H | BENNETT | ID=center of west end of concrete drainage channel on north side of SR 20 |
| HV-09 | 05013I | COMPASS LAKE | ID=corner fence post in SE quadrant of intersection of Scot Road and County Line Road |
| HV-10 | 05013J | FOUNTAIN | ID=lone tall wooden post on east side of Sorrelss Drive at Webber Road |
| HV-11 | 05013K | YOUNGSTOWN | ID=easterly fence post for gate on north side of Scotts Ferry Road, at angle in fence |
| HV-12 | 05013L | JUNIPER CREEK | ID=NW corner of easterly concrete drainage structure under field road north on north side of SR 20 |
| HV-13 | 05013M | FRINK | ID=SW edge of pavement at NW corner of turnout for field road to the SW on SW side of CR 575 |
| HV-14 | 05013N | FRINK | ID=NE corner of basketball court on west side of CR 73 |
| HV-15 | 05013Y | TENMILE SWAMP | ID=SE corner of concrete wall on north side of SR 22 |
| HV-16 | 05013P | DEAD LAKE | ID=NE corner of concrete drive at west edge of Byrd Parker Road |
| HV-17 | 05013Q | WETAPPO CREEK | ID=NW corner of asphalt turnout to field road on north side of CR 386 |
| HV-18 | 05013R | POPLAR HEAD | ID=intersection of concrete sidewalks at school, west edge of N-S sidewalk and north edge of E-W sidewalk, at west side of asphalt parking lot |
| HV-19 | 05013S | WHITE CITY | ID=SE corner of faded stop bar for CR 387 (Doc Whitfield Road) at south side of SR 71 |
| HV-20 | 05013T | OVERSTREET | ID=SW corner of asphalt turnout for field road on south side of CR 386 |
| HV-21 | 05013U | PORT SAINT JOE | ID=NE corner of painted white curb around grave site, east side of N-S sand road and south of asphalt road at north edge of cemetery |
| HV-22 | 05013V | CAPE SAN BLAS | ID=NW corner of concrete drive apron to South Gulf County VFD, south side of Cape San Blas Road (CR 30E) |
| HV-23 | HV-23 | JACKSON RIVER | ID=lone post for telephone pedestal on SW side of sand road (unnamed) |
| HV-24 | 05013X | WEST PASS | ID=lone post in SE quadrant of intersection of CR 385 (Tilton Road) and railroad |
| KPFN | KPFN | PANAMA CITY | ABGPS base station |
| PANAMA CORS ARP | PNCY | PANAMA CITY | CORS |
| PARISH RESET | BE3768 | HINSONS CROSSROADS | H/V control |
| X 88 | BE0864 | FOUNTAIN | H/V control |

The map below in figure 1 shows the distribution of the control stations, shown as purple triangles and the new stations, shown as blue circles.

Figure 1 - Project Location

# GPS OBSERVATIONS

The network was observed in a radial configuration. Base receivers (2) were established on random points, and run throughout the observations in each area. Due to the large area covered, multiple base locations were used. The observations were made on days 39 and 40 of 2005, using four Trimble dual frequency receivers. Additionally the CORS data from PNCY was downloaded for day 349 of 2004 to tie the ABGPS base station into the photo control network. The table below summarizes the occupations (duration in minutes, HI in meters):

| GPSID | UTC start | UTC stop | Dur. | HI | Filename |
| --- | --- | --- | --- | --- | --- |
| 05013D | 2/8/2005 14:52:45 | 2/8/2005 15:07:30 | 15 | 2 | 36650390.dat |
| 05013A | 2/8/2005 15:41:00 | 2/8/2005 15:56:15 | 15 | 2 | 36650391.dat |
| 05013B | 2/8/2005 16:26:00 | 2/8/2005 16:52:00 | 26 | 2.5 | 36650392.dat |
| 05013C | 2/8/2005 17:25:00 | 2/8/2005 17:50:30 | 25 | 2.5 | 36650393.dat |
| 05013G | 2/8/2005 18:26:45 | 2/8/2005 18:55:15 | 29 | 2.5 | 36650394.dat |
| 05013F | 2/8/2005 19:24:45 | 2/8/2005 20:00:30 | 36 | 2.5 | 36650395.dat |
| 05013R | 2/8/2005 20:29:00 | 2/8/2005 21:01:15 | 32 | 2.5 | 36650396.dat |
| BE3768 | 2/8/2005 21:22:45 | 2/8/2005 21:50:15 | 28 | 2 | 36650397.dat |
| 05013E | 2/8/2005 22:07:15 | 2/8/2005 22:32:30 | 25 | 2.5 | 36650398.dat |
| 05013H | 2/8/2005 20:15:30 | 2/8/2005 20:30:00 | 15 | 2.5 | 37950390.dat |
| 05013I | 2/8/2005 20:54:00 | 2/8/2005 21:22:00 | 28 | 1.24 | 37950391.dat |
| 05013J | 2/8/2005 21:41:15 | 2/8/2005 21:56:15 | 15 | 2.31 | 37950392.dat |
| BE0864 | 2/8/2005 22:15:30 | 2/8/2005 22:30:00 | 15 | 1.5 | 37950393.dat |
| 05013K | 2/8/2005 22:46:00 | 2/8/2005 23:03:15 | 17 | 3 | 37950394.dat |
| 05013AB | 2/8/2005 19:52:45 | 2/8/2005 23:11:15 | 199 | 1.52 | 97570390.dat |
| 05013AA | 2/8/2005 13:51:45 | 2/8/2005 23:10:00 | 559 | 0 | 99810390.DAT |
| 05013M | 2/9/2005 14:24:00 | 2/9/2005 14:50:15 | 26 | 2.5 | 36650400.dat |
| 05013N | 2/9/2005 15:06:45 | 2/9/2005 15:30:00 | 24 | 2.5 | 36650401.dat |
| 05013P | 2/9/2005 15:56:45 | 2/9/2005 16:16:45 | 20 | 2.5 | 36650402.dat |
| AS0900 | 2/9/2005 16:52:00 | 2/9/2005 17:22:15 | 30 | 2 | 36650403.dat |
| 05013Q | 2/9/2005 17:45:30 | 2/9/2005 18:00:30 | 15 | 2.5 | 36650404.dat |
| 05013T | 2/9/2005 18:17:00 | 2/9/2005 18:41:30 | 24 | 2.5 | 36650405.dat |
| 05013O | 2/9/2005 19:35:00 | 2/9/2005 19:51:00 | 16 | 2.5 | 36650406.dat |
| 05013Y | 2/9/2005 19:58:45 | 2/9/2005 20:13:15 | 15 | 1.4 | 36650407.dat |
| 05013L | 2/9/2005 14:02:00 | 2/9/2005 14:17:00 | 15 | 3 | 37950400.dat |
| 05013V | 2/9/2005 16:03:45 | 2/9/2005 16:19:00 | 16 | 3 | 37950401.dat |
| 05013X | 2/9/2005 16:52:30 | 2/9/2005 17:15:00 | 23 | 1.38 | 37950402.dat |
| AS0895 | 2/9/2005 17:43:45 | 2/9/2005 18:15:30 | 32 | 2 | 37950403.dat |
| 05013W | 2/9/2005 19:16:45 | 2/9/2005 19:47:00 | 31 | 3 | 37950404.dat |
| 05013S | 2/9/2005 20:03:15 | 2/9/2005 20:18:00 | 15 | 3 | 37950405.dat |
| 05013AB | 2/9/2005 13:36:30 | 2/9/2005 14:32:30 | 56 | 1.52 | 97570400.dat |
| 05013U | 2/9/2005 15:42:00 | 2/9/2005 20:41:00 | 299 | 1.343 | 97570401.dat |
| 05013AC | 2/9/2005 13:50:00 | 2/9/2005 20:23:30 | 393 | 0 | 99810400.DAT |
| PNCY | 2/8/2005 13:51:00 | 2/8/2005 23:11:00 | 560 | 0 | PNCY039.05O |
| PNCY | 2/9/2005 13:36:00 | 2/9/2005 20:41:00 | 425 | 0 | PNCY040.05O |
| KPFN | 12/14/2004 15:23:00 | 12/14/2004 20:38:00 | 315 | 0 | KPFN349.04O |
| PNCY | 12/14/2004 15:23:00 | 12/14/2004 20:38:00 | 315 | 0 | PNCY349.04O |

# DATA PROCESSING

The GPS data collected was downloaded to a PC and processed using the WAVE (Weighted Ambiguity Vector Estimator) processor in Trimble Geomatics Office (TGO) version 1.6. The single baseline method was used. In general, all lines radiating from the base receivers to the nearby stations were processed. In addition, the baselines between the bases and the CORS were processed and included to strengthen the network. The precise ephemeris was used. All solutions utilized the integer bias fixed solution. The table below summarizes the processing results:

| From | To | UTC start | Dur. | Length | Ratio | Var. | RMS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 05013AB | 05013H | 2/8/2005 20:15:30 | 15 | 15219 | 7.29 | 0.8 | 0.013 |
| 05013AB | 05013I | 2/8/2005 20:54:00 | 28 | 24296 | 19.55 | 0.7 | 0.011 |
| 05013AB | 05013J | 2/8/2005 21:41:15 | 15 | 14320 | 11.42 | 1.4 | 0.013 |
| 05013AB | BE0864 | 2/8/2005 22:15:30 | 15 | 5074 | 56.95 | 0.7 | 0.009 |
| 05013AB | 05013K | 2/8/2005 22:46:00 | 17 | 2031 | 16.99 | 2.4 | 0.006 |
| 05013AB | PNCY | 2/8/2005 19:52:45 | 199 | 27722 | 7.21 | 0.6 | 0.01 |
| 05013AA | 05013D | 2/8/2005 14:52:45 | 15 | 5189 | 29.92 | 1 | 0.011 |
| 05013AA | 05013A | 2/8/2005 15:41:00 | 15 | 13002 | 7.55 | 2.1 | 0.018 |
| 05013AA | 05013B | 2/8/2005 16:26:00 | 26 | 22895 | 21.77 | 1.4 | 0.014 |
| 05013AA | 05013C | 2/8/2005 17:25:00 | 25 | 18883 | 18.06 | 0.8 | 0.012 |
| 05013AA | 05013G | 2/8/2005 18:26:45 | 29 | 25473 | 15.44 | 1 | 0.013 |
| 05013AA | 05013F | 2/8/2005 19:24:45 | 36 | 29450 | 12.63 | 1.5 | 0.014 |
| 05013AA | 05013R | 2/8/2005 20:29:00 | 32 | 27049 | 14.41 | 1 | 0.011 |
| 05013AA | BE3768 | 2/8/2005 21:22:45 | 28 | 19200 | 40.2 | 0.9 | 0.01 |
| 05013AA | 05013E | 2/8/2005 22:07:15 | 25 | 12860 | 3.18 | 2.3 | 0.017 |
| 05013AA | 05013AB | 2/8/2005 19:52:45 | 198 | 59366 | 4.45 | 0.9 | 0.012 |
| 05013AA | PNCY | 2/8/2005 13:51:45 | 559 | 52492 | 13.7 | 0.9 | 0.011 |
| 05013AB | 05013L | 2/9/2005 14:02:00 | 15 | 15783 | 21.05 | 0.6 | 0.007 |
| 05013AB | PNCY | 2/9/2005 13:36:30 | 56 | 27722 | 46.34 | 0.4 | 0.007 |
| 05013U | 05013V | 2/9/2005 16:03:45 | 16 | 12335 | 10.42 | 0.9 | 0.011 |
| 05013U | 05013X | 2/9/2005 16:52:30 | 23 | 19631 | 19.57 | 0.7 | 0.011 |
| 05013U | AS0895 | 2/9/2005 17:43:45 | 32 | 25411 | 10.27 | 0.7 | 0.01 |
| 05013U | 05013W | 2/9/2005 19:16:45 | 31 | 20800 | 10.84 | 1.2 | 0.012 |
| 05013U | 05013S | 2/9/2005 20:03:15 | 15 | 14802 | 12.97 | 1 | 0.012 |
| 05013U | PNCY | 2/9/2005 15:42:00 | 299 | 59992 | 3.52 | 0.8 | 0.011 |
| 05013AC | 05013M | 2/9/2005 14:24:00 | 26 | 23962 | 17.6 | 0.8 | 0.009 |
| 05013AC | 05013N | 2/9/2005 15:06:45 | 24 | 14158 | 20.39 | 0.6 | 0.009 |
| 05013AC | 05013P | 2/9/2005 15:56:45 | 20 | 9296 | 65.05 | 1.4 | 0.012 |
| 05013AC | AS0900 | 2/9/2005 16:52:00 | 30 | 22156 | 9.51 | 1.6 | 0.015 |
| 05013AC | 05013Q | 2/9/2005 17:45:30 | 15 | 8438 | 14.09 | 1 | 0.012 |
| 05013AC | 05013T | 2/9/2005 18:17:00 | 24 | 18385 | 12.93 | 1.2 | 0.014 |
| 05013AC | 05013O | 2/9/2005 19:35:00 | 16 | 10255 | 21.14 | 0.9 | 0.011 |
| 05013AC | 05013Y | 2/9/2005 19:58:45 | 15 | 11422 | 33.64 | 0.5 | 0.009 |
| 05013AC | 05013AB | 2/9/2005 13:50:00 | 42 | 30113 | 33.81 | 0.4 | 0.007 |
| 05013AC | 05013U | 2/9/2005 15:42:00 | 281 | 38457 | 2.78 | 1 | 0.013 |
| 05013AC | PNCY | 2/9/2005 13:50:00 | 393 | 42373 | 10.4 | 1 | 0.011 |
| PNCY | KPFN | 12/14/2004 15:23:00 | 315 | 61 | 45.73 | 0.8 | 0.003 |

In general, there are several indicators of baseline quality. For integer bias fixed solutions, the ratio should be larger than 3.0, the rms should be below 0.02, and the variance should be near unity. All baselines exhibit good results.

There was one repeat baseline, which is a baseline that has been measured in more than one independent session. The Earth Centered Earth Fixed vector differences were rotated into a local horizon system for analysis (sorted by increasing difference in the vertical component, all values in meters):

From       To         V1        V2        Delta N   Delta E   Delta U  Length

05013AB    PNCY       36        37        -0.001    +0.004    +0.027   27723

As can be seen, the repeat baseline agrees very well in the horizontal component and to better than 0.03 m in the vertical component.

LEAST SQUARES ADJUSTMENTS

The data was adjusted using GEOLAB, a least squares adjustment program from Microsearch Corp. The processed baselines were parsed to form an input file. No scaling of the apriori baseline statistics was done. Station errors (HI and centering) of 0.005 m were also included. Geoid separations for each station were interpolated using the GEOID03 model.

The first adjustment was a minimally constrained adjustment which held CORS station PNCY fixed in all three dimensions (latitude and longitude and orthometric height). The estimated variance factor was 0.60. The graph below in figure 2 shows the horizontal and vertical residuals versus baseline length for this adjustment.

Figure 2 - residuals

As can be seen, the vertical residuals are within the range of –0.015 m to +0.015 m. The horizontal residuals are in the range of 0.000 to 0.005 m. The misclosures at the other control stations were as follows:

PID		Azimuth		Distance	 Ortho H	 Ellip H

AS0895         131ø	 	0.020 m		-0.025 m	-0.031 m

AS0900         142ø	 	0.032 m		+0.018 m	-0.013 m

BE0864         253ø	 	0.019 m		-0.059 m	-0.142 m

BE3768         146ø	 	0.003 m		-0.059 m	-0.073 m

PNCY             0ø	 	0.000 m	        		+0.000 m

All of the control checked within expected tolerances with the exception of the orthometric height at benchmark FLGPS 13 AZ MK (PID AS0900). The difference was not considered large enough to not constrain this benchmark in the final adjustment. The benchmark appeared to be undisturbed in the field; therefore it was decided to constrain this benchmark in the final adjustment. All benchmarks occupied were constrained in the final adjustment. The output from this adjustment is included in appendix A.

The final adjustment fixed the CORS station PNCY horizontally (latitude and longitude). The other stations, APALPORT AZ MK (PID AS0895), FLGPS 13 AZ MK (PID AS0900), PARISH RESET (PID BE3768), and X 88 (PID BE0864) were constrained in all three dimensions. The estimated variance factor for this adjustment was 3.22. The graph below in figure 3 shows the horizontal and vertical residuals for this adjustment.

Figure 3 - residuals

The residuals from this adjustment are within the ± 0.03m range. The output from this adjustment is included in appendix B. This adjustment provided the final adjusted coordinates for the network.

The table below shows the 2-D and 1-D Station Confidence Regions (95.000 percent, in meters) for the stations in the network:

| STATION | MAJOR SEMI-AXIS | AZIMUTH | MINOR SEMI-AXIS | VERTICAL |
| --- | --- | --- | --- | --- |
| 05013A | 0.041 | 112 | 0.038 | 0.044 |
| 05013AA | 0.019 | 037 | 0.019 | 0.020 |
| 05013AB | 0.015 | 008 | 0.015 | 0.019 |
| 05013AC | 0.017 | 149 | 0.017 | 0.018 |
| 05013B | 0.038 | 118 | 0.037 | 0.036 |
| 05013C | 0.037 | 155 | 0.037 | 0.034 |
| 05013D | 0.037 | 103 | 0.037 | 0.039 |
| 05013E | 0.038 | 014 | 0.037 | 0.043 |
| 05013F | 0.037 | 156 | 0.037 | 0.036 |
| 05013G | 0.037 | 172 | 0.037 | 0.035 |
| 05013H | 0.036 | 156 | 0.035 | 0.035 |
| 05013I | 0.035 | 149 | 0.035 | 0.032 |
| 05013J | 0.037 | 042 | 0.036 | 0.043 |
| 05013K | 0.035 | 004 | 0.035 | 0.032 |
| 05013L | 0.036 | 005 | 0.035 | 0.034 |
| 05013M | 0.036 | 146 | 0.036 | 0.036 |
| 05013N | 0.036 | 115 | 0.036 | 0.035 |
| 05013O | 0.036 | 178 | 0.036 | 0.036 |
| 05013P | 0.038 | 107 | 0.037 | 0.038 |
| 05013Q | 0.037 | 001 | 0.036 | 0.037 |
| 05013R | 0.037 | 153 | 0.037 | 0.034 |
| 05013S | 0.037 | 169 | 0.037 | 0.037 |
| 05013T | 0.037 | 167 | 0.036 | 0.035 |
| 05013U | 0.019 | 165 | 0.019 | 0.020 |
| 05013V | 0.038 | 111 | 0.037 | 0.036 |
| 05013W | 0.037 | 155 | 0.037 | 0.037 |
| 05013X | 0.037 | 134 | 0.037 | 0.034 |
| 05013Y | 0.036 | 176 | 0.036 | 0.033 |
| HV-23 | 0.057 | 132 | 0.040 | 0.051 |
| KPFN | 0.031 | 117 | 0.031 | 0.032 |
| PNCY | 0.000 | 000 | 0.000 | 0.019 |

SUMMARY

A geodetic control network was established for the NW Greenway project area in Bay, Calhoun, Franklin, Gulf, Walton and Washington Counties, Florida for the purpose of controlling digital aerial imagery. The accuracy of the adjusted coordinates is ±0.04 m. The network configuration is shown in figure 4.

Figure 4 - network diagram

ADJUSTED COORDINATES – NAD 1983 1999/NAVD 1988 meters

| Station Name | GPSID | USGS Quad | Latitude | Longitude | Ellip H | Ortho H |
| --- | --- | --- | --- | --- | --- | --- |
| HV-01 | 05013A | DE FUNIAK SPRINGS EAST | 30º40'35.67925" N | 86º05'11.66819" W | 26.243 | 53.798 |
| BASE | 05013AA | REDBAY | 30º35'28.18428" N | 85º59'37.07286" W | 29.689 | 57.354 |
| BASE | 05013AB | YOUNGSTOWN | 30º20'54.54825" N | 85º26'33.14670" W | -3.938 | 23.988 |
| BASE | 05013AC | DEAD LAKE | 30º08'16.15423" N | 85º14'42.09305" W | -12.830 | 14.911 |
| HV-02 | 05013B | PORTLAND | 30º30'58.35010" N | 86º12'57.55119" W | -9.790 | 17.727 |
| HV-03 | 05013C | BUNKER | 30º25'17.20130" N | 86º00'37.19668" W | -21.253 | 6.419 |
| HV-04 | 05013D | ROCK HILL | 30º33'06.39556" N | 86º01'22.33986" W | 16.987 | 44.637 |
| HV-05 | 05013E | MILLERS FERRY | 30º34'47.34554" N | 85º51'36.67754" W | -7.783 | 19.973 |
| HV-06 | 05013F | VERNON | 30º30'48.10852" N | 85º42'00.53587" W | -4.925 | 22.929 |
| HV-07 | 05013G | RED HEAD | 30º26'23.05650" N | 85º47'38.45209" W | 0.366 | 28.171 |
| HV-08 | 05013H | BENNETT | 30º25'39.43146" N | 85º34'18.97734" W | -7.361 | 20.547 |
| HV-09 | 05013I | COMPASS LAKE | 30º34'03.35564" N | 85º26'51.69883" W | 40.875 | 68.887 |
| HV-10 | 05013J | FOUNTAIN | 30º28'18.84766" N | 85º23'54.76444" W | 25.433 | 53.464 |
| HV-11 | 05013K | YOUNGSTOWN | 30º21'08.58678" N | 85º25'18.83178" W | -1.735 | 26.204 |
| HV-12 | 05013L | JUNIPER CREEK | 30º25'21.57170" N | 85º18'08.50410" W | 10.667 | 38.744 |
| HV-13 | 05013M | FRINK | 30º19'43.84146" N | 85º07'42.66384" W | -0.728 | 27.312 |
| HV-14 | 05013N | FRINK | 30º15'55.87588" N | 85º14'31.86357" W | -7.022 | 20.91 |
| EXTRA | 05013O | TENMILE SWAMP | 30º08'02.34075" N | 85º21'04.91840" W | -10.861 | 16.876 |
| HV-16 | 05013P | DEAD LAKE | 30º08'07.30731" N | 85º08'54.90154" W | -20.673 | 7.069 |
| HV-17 | 05013Q | WETAPPO CREEK | 30º03'44.88213" N | 85º15'26.91975" W | -18.551 | 9.090 |
| HV-18 | 05013R | POPLAR HEAD | 30º37'32.41999" N | 85º42'51.77834" W | -14.912 | 12.903 |
| HV-19 | 05013S | WHITE CITY | 29º54'36.96607" N | 85º12'17.75572" W | -21.463 | 5.953 |
| HV-20 | 05013T | OVERSTREET | 29º59'40.98750" N | 85º20'29.08871" W | -23.674 | 3.882 |
| HV-21 | 05013U | PORT SAINT JOE | 29º47'30.95632" N | 85º16'33.25344" W | -19.998 | 7.221 |
| HV-22 | 05013V | CAPE SAN BLAS | 29º41'06.42251" N | 85º18'42.06108" W | -24.925 | 2.159 |
| ECC STA | 05013W | JACKSON RIVER | 29º51'20.86422" N | 85º04'24.78319" W | -25.622 | 1.667 |
| HV-24 | 05013X | WEST PASS | 29º44'20.75698" N | 85º04'55.74901" W | -24.377 | 2.776 |
| HV-15 | 05013Y | TENMILE SWAMP | 30º08'05.82501" N | 85º21'48.69204" W | -11.239 | 16.499 |
| APALPORT AZ MK | AS0895 | WEST PASS | 29º43'37.18240" N | 85º01'26.14750" W | -21.676 | 5.455 |
| FLGPS 13 AZ MK | AS0900 | FORBES ISLAND | 29º58'08.28296" N | 85º07'19.48671" W | -22.631 | 4.865 |
| X 88 | BE0864 | FOUNTAIN | 30º23'36.79814" N | 85º25'59.99918" W | 0.936 | 28.895 |
| PARISH RESET | BE3768 | HINSONS CROSSROADS | 30º39'05.94890" N | 85º48'21.54147" W | 2.586 | 30.342 |
| KPFN | KPFN | PANAMA CITY | 30º12'17.77117" N | 85º40'39.71472" W | -22.213 | 5.530 |
| PANAMA CORS ARP | PNCY | PANAMA CITY | 30º12'16.51768" N | 85º40'41.47891" W | -17.171 | 10.571 |
| HV-23 | HV-23 | JACKSON RIVER | 29º51'20.86900" N | 85º04'24.78928" W | -25.622 | 1.667 |

Note: Ellipsoidal heights are based on NAVD 1988 GPS derived orthometric heights plus GEOID03.

Florida West State Plane Coordinates – meters & US Survey FT

UTM Zone 16 Coordinates – meters

| Station Name | GPSID | SPC N m | SPC E m | Ortho H | SPC N ft | SPC E ft | Ortho H ft | UTM N m | UTM E m |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASE | 05013AA | 177317.314 | 456754.468 | 57.354 | 581748.554 | 1498535.284 | 188.169 | 3384725.128 | 596483.896 |
| BASE | 05013AB | 149851.346 | 509382.427 | 23.988 | 491637.291 | 1671198.846 | 78.701 | 3358429.546 | 649694.823 |
| BASE | 05013AC | 126358.886 | 528218.968 | 14.911 | 414562.445 | 1732998.398 | 48.921 | 3335357.242 | 669042.024 |
| EXTRA | 05013O | 126005.288 | 517970.390 | 16.876 | 413402.349 | 1699374.521 | 55.367 | 3334779.148 | 658803.831 |
| ECC STA | 05013W | 95002.046 | 544582.652 | 1.667 | 311685.879 | 1786684.917 | 5.469 | 3304364.260 | 686087.415 |
| APALPORT AZ MK | AS0895 | 80702.151 | 549311.468 | 5.455 | 264770.307 | 1802199.375 | 17.897 | 3290169.553 | 691126.291 |
| PARISH RESET | BE3768 | 183801.914 | 474826.631 | 30.342 | 603023.446 | 1557827.039 | 99.547 | 3391605.109 | 614404.285 |
| KPFN | KPFN | 134149.746 | 486610.293 | 5.530 | 440122.958 | 1596487.270 | 18.143 | 3342233.285 | 627275.682 |
| PANAMA CORS ARP | PNCY | 134111.639 | 486562.713 | 10.571 | 439997.936 | 1596331.168 | 34.682 | 3342194.148 | 627228.956 |
| HV-24 | 05013X | 82071.135 | 543685.512 | 2.776 | 269261.715 | 1783741.551 | 9.108 | 3291416.167 | 685471.601 |
| HV-23 | HV-23 | 95002.194 | 544582.489 | 1.667 | 311686.365 | 1786684.383 | 5.469 | 3304364.404 | 686087.249 |
| HV-22 | 05013V | 76223.373 | 521439.556 | 2.159 | 250076.183 | 1710756.277 | 7.083 | 3285086.788 | 663357.948 |
| HV-21 | 05013U | 88038.938 | 524983.104 | 7.221 | 288841.082 | 1722382.067 | 23.691 | 3296976.101 | 666643.709 |
| HV-20 | 05013T | 110561.364 | 518815.397 | 3.882 | 362733.408 | 1702146.848 | 12.736 | 3319357.981 | 659986.555 |
| HV-19 | 05013S | 101111.194 | 531925.876 | 5.953 | 331728.976 | 1745160.145 | 19.531 | 3310196.389 | 673300.042 |
| HV-18 | 05013R | 180824.834 | 483575.191 | 12.903 | 593256.143 | 1586529.606 | 42.333 | 3388822.500 | 623214.613 |
| HV-17 | 05013Q | 118014.168 | 526963.793 | 9.090 | 387184.816 | 1728880.378 | 29.823 | 3326987.030 | 667969.880 |
| HV-16 | 05013P | 126029.694 | 537509.22 | 7.069 | 413482.421 | 1763478.166 | 23.192 | 3335231.717 | 678337.480 |
| HV-15 | 05013Y | 126121.381 | 516799.666 | 16.499 | 413783.231 | 1695533.571 | 54.130 | 3334869.554 | 657630.889 |
| HV-14 | 05013N | 140512.418 | 528584.882 | 20.910 | 460997.825 | 1734198.900 | 68.602 | 3349515.451 | 669097.256 |
| HV-13 | 05013M | 147466.136 | 539560.784 | 27.312 | 483811.815 | 1770209.006 | 89.606 | 3356709.040 | 679917.850 |
| HV-12 | 05013L | 157970.413 | 522917.063 | 38.744 | 518274.597 | 1715603.731 | 127.113 | 3366844.247 | 663046.662 |
| HV-11 | 05013K | 150267.387 | 511370.538 | 26.204 | 493002.252 | 1677721.507 | 85.971 | 3358889.198 | 651673.153 |
| HV-10 | 05013J | 163498.068 | 513719.923 | 53.464 | 536409.911 | 1685429.447 | 175.406 | 3372167.495 | 653730.349 |
| HV-09 | 05013I | 174144.616 | 509088.880 | 68.887 | 571339.461 | 1670235.767 | 226.007 | 3382708.329 | 648865.839 |
| HV-08 | 05013H | 158733.215 | 497024.745 | 20.547 | 520777.223 | 1630655.351 | 67.411 | 3367036.346 | 637145.892 |
| HV-07 | 05013G | 160297.828 | 475707.167 | 28.171 | 525910.457 | 1560715.930 | 92.424 | 3368131.014 | 615802.477 |
| HV-06 | 05013F | 168360.626 | 484808.725 | 22.929 | 552363.154 | 1590576.625 | 75.226 | 3376390.824 | 624722.569 |
| HV-05 | 05013E | 175899.637 | 469536.587 | 19.973 | 577097.392 | 1540471.286 | 65.528 | 3383589.839 | 609291.032 |
| HV-04 | 05013D | 172988.529 | 453891.997 | 44.637 | 567546.532 | 1489143.993 | 146.447 | 3380335.479 | 593718.211 |
| HV-03 | 05013C | 158525.646 | 454903.637 | 6.419 | 520096.224 | 1492463.016 | 21.060 | 3365902.034 | 595047.571 |
| HV-02 | 05013B | 169309.117 | 435306.131 | 17.727 | 555474.995 | 1428166.865 | 58.159 | 3376249.019 | 575223.023 |
| HV-01 | 05013A | 186906.036 | 447973.151 | 53.798 | 613207.553 | 1469725.246 | 176.502 | 3394115.349 | 587495.839 |
| FLGPS 13 AZ MK | AS0900 | 107571.135 | 539962.220 | 4.865 | 352922.965 | 1771526.050 | 15.961 | 3316830.347 | 681193.770 |
| X 88 | BE0864 | 154840.007 | 510308.545 | 28.895 | 508004.256 | 1674237.285 | 94.800 | 3363437.000 | 650510.847 |